Flanged Diffuser - Chemical Injection Assembly

Section 11XXX

1.   Type: Flanged Multi-Orifice Chemical Diffuser

2.  Connection:  Designed to fit inside a __ inch diameter schedule 40 flanged additive port.  Diffuser flange to be sandwiched between additive port flange and drilled & tapped blind flange with 1/8” Teflon gaskets.

3.  Construction:

Chemical diffuser assembly shall come complete with diffuser assembly and flange gaskets. Diffuser tube shall be constructed of (material of construction).  Diffuser tube shall have  adequate length to extend across the pipe diameter and have additional length of (2 injector pipe diameters) .  Chemical diffuser to be supported on the opposing side (180º) by an additional additive port with blind flange assembly & teflon gasket.

Both additive ports shall be in alignment to allow for easy insertion and withdrawal of the chemical diffuser without binding or bending of the diffuser.  The nominal __ inch diameter diffuser solution tube should be a diffuser style with multiple holes for even distribution of the injected chemical. and situated at least (2 pipe diameters)  inches from the pipeline walls.  All wetted components shall be compatible with the chemical services.   Model “_____” manufactured by Inyo Process or equal that has been preapproved by engineer before the bid